(QUAKER T500 SH SINGLE-HUNG WINDOW)

SECTION 08 51 13 (08520) - ALUMINUM WINDOWS

PART 1	  GENERAL

1.01	SUMMARY

A. 	Section Includes:

1. 	Operating and Fixed Aluminum Window Units

a. Single-Hung Aluminum Windows.

2. 	Glass and Glazing for Aluminum Windows.

3. 	Wood Blocking, Shims, Anchors, Clips, and all accessories necessary for a complete installation furnished and installed.

4. 	All aluminum trim and closure pieces

5.  Installation labor, tools, equipment, and services necessary for installation of Aluminum Windows.

B.	Related Sections:

      - Section 07 62 00 (07620) -  Sheet Metal Flashing and Trim

      - Section 07 92 00 (07920) -  Joint Sealants

      - Section 08 13 73 (08130) – Sliding Aluminum Doors and Frames

      - Section 08 41 13 (08410) – Aluminum Entrances and Storefront

      - Section 08 80 00 (08800) - Glazing

- 1.02	REFERENCES

A.	 Aluminum Association (AA)

      - DAF-45 – “Designation System for Aluminum Finishes”

- B.	Fenestration and Glazing Industry Alliance (FGIA) (a.k.a.)American Architectural Manufacturers Association (AAMA):

      - 101 – “Voluntary Performance Specification for Windows, Skylights and Glass Doors”

      - 502 – “Voluntary Specification for Field Testing of Newly Installed Fenestration Products”

      - 513 – “Voluntary Specification for Standard Laboratory Test Method For Determination Of Forces And Motions Required To Activate Operable Parts Of Operable Windows And Doors In Accessible Spaces”

      - 611 – “Voluntary Specification for Anodized Architectural Aluminum”

      - 1503 – “Voluntary Test Method for Thermal Transmittance and Condensation Resistance of Windows, Doors and Glazed Wall Sections”

      - 2400 – “Voluntary Specification for Installation of Windows with a Mounting Flange in Stud Frame Construction”

      - 2604 – “Voluntary Specification, Performance Requirements and Test Procedures for High Performance Organic Coatings on Aluminum Extrusions and Panels”

      - 2605 – “Voluntary Specification, Performance Requirements and Test Procedures for Superior Performance Organic Coatings on Aluminum Extrusions and Panels”

      - CW-10 – “Care and Handling  of Architectural Aluminum from Shop to Site”

- C.  American National Standards Institute (ANSI) Publications

- Z97.1 – “Performance Specifications and Methods of Test for Safety Glazing Materials Used in Buildings”

- D.  ASTM International (ASTM) Publications:

- C518 – “Standard Test Method for Steady-State Thermal Transmission Properties by Means of the Heat Flow Meter Apparatus”

- C1036 – “Standard Specifications for Flat Glass”

- C1048 – “Standard Specifications for Heat-Treated Flat Glass – Kind HS, Kind FT Coated and Uncoated Glass”

- D3985 – “Standard Test Method for Oxygen Gas Transmission Rate Through Plastic Film and Sheeting Using a Coulometric Sensor”

- E90 – “Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements”

- E283 – “Standard Test Method for Determining the Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen”

- E330 – “Standard Test Method for Structural Performance of Exterior Windows, Curtain Walls, and Doors By Uniform Static Air Pressure Difference”

- E331 – “Standard Test Method for Water Penetration of Exterior Windows, Curtain Walls, and Doors By Uniform Static Air Pressure Difference”

- E413 – “Classification for Rating Sound Insulation”

- E547 – “Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Cyclic Static Air Pressure Differential”

- E774 – “Standard Specification for Sealed Insulating Glass Units”

- F588 -  “Standard Test Methods for Measuring the Forced Entry Resistance of Window Assemblies, Excluding Glazing Impact”

- F1249 – “Standard Test Method for Water Vapor Transmission Rate Through Plastic Film and Sheeting Using a Modulated Infrared Sensor”

- E.  National Glass Association (NGA):

- 1.	“GANA Glazing Manual”

- F.  Federal Specifications (FS) Publications:

- 1.	FS-RR-W-365A “Wire Fabric (Insect Screening)”

- G.  Insulating Glass Certification Council (IGCC)

- H.   National Fenestration Ratings Council (NFRC)

- I.  Screen Manufacturers Association (SMA) Publications:

- 1.  	1004 “Specifications for Aluminum Tubular Frame Screens for Windows”

- J.  U.S. Consumer Product Safety Commission (CPSC) Publications:

- 1. 	16 CFR Part 1201 “Safety Standard For Architectural Glazing Materials”

- K.  Window and Door Manufacturers Association (WDMA) Publications:

- 1.	FGIA/AAMA/WDMA  101/I.S.2/NAFS “Voluntary Performance				Specification for Windows, Skylights and Glass Doors”

- 2.  	FGIA/AAMA/WDMA/CSA  101/I.S.2/A440 “Standard/Specification for 			Windows, Doors and Unit Skylights”

1.03	SUBMITTALS

A. 	Submit “Letter of Conformance” in accordance with Section 01 33 00 (01330) – with the following supporting data:

- 1. 	Product data for each type of aluminum window specified, including standard construction details, dimensions of individual components, profiles, finishes, hardware, and accessories.

2. 	Shop drawings for each type of window specified, including ¼-inch scale wall elevations, typical unit elevations at ¾-inch scale details, full size details of typical composite members and the following:

- Panning Details

- Flashing and drainage details.

- Mullion details, including reinforcement and stiffeners.

- Joinery details.

3.  Samples:  Provide full-size or partial-size sample of window illustrating		glazing system, quality of construction and finish.

4.  Product certificates signed by the window manufacturer certifying that 	window units comply with specified performance requirements.

5.	Submit certified independent laboratory test reports verifying compliance 	with all test requirements of 1.05 PERFORMANCE				REQUIREMENTS as requested by Architect.

1.04	DEFINITIONS

A. 	Performance grade number, included as part of the FGIA/AAMA/WDMA/CSA 101/I.S.2/A440 product designation code, is actual design pressure in pounds force per square foot used to determine structural test pressure and water test pressure.

1.05    PERFORMANCE REQUIREMENTS

A.   Certify that windows have been tested in accordance with American			       Architectural Manufacturers Association (FGIA/AAMA/WDMA) Specification		       for Performance Class specified complying with the following performance 		       standards:

### FGIA/AAMA/WDMA/CSA 101/I.S.2/A440 Performance Requirements:  Provide aluminum windows of the performance class and grade indicated that comply with FGIA/AAMA/WDMA/CSA 101/I.S.2/A440.

- Performance Class:	H-AW

- Performance Grade:	50

- Uniform Structural Properties (ASTM E330):	Pressure acting inward and outward.  Window to be operable with permanent deformation at a maximum of 1/175 of its span, when tested at a static air pressure difference of the following:

- Class H-AW-50:  75.0 PSF

- Water Resistance (ASTM E331 and ASTM E547):	No water penetration at test pressure indicated.

- Class H-AW-50:  10.00 PSF

- Air Leakage (ASTM E283):

- Single-Hung Windows:  Maximum 0.3 CFM per sq./ft. of total exterior surface area, when tested at a static air pressure differential of 6.2 PSF minimum.

- Fixed Windows:  Maximum 0.10 CFM per sq./ft. of total exterior surface area, when tested at a static air pressure differential of 6.2 PSF minimum.

B.  Project Wind Loads:

Please specify design wind pressure, both positive and negative required by the governing building code, as calculated from ANSI A58.1, or as determined by  boundary layer wind tunnel testing.  Minimum for positive and negative pressure = 10.0 p.s.f.

- The system shall be designed to withstand the following loads with respect to the plane of the wall:

- Positive pressure of ___ p.s.f. at non corner zones.

- Negative pressure of ___ p.s.f. at non corner zones.

- Negative pressure of ___ p.s.f. at corner zones.

1.06	QUALITY ASSURANCE

- A.  All window units shall be manufactured by a single source.

- 1.  All windows in any one project must be by the same manufacturer and 	with comparable frame depth, profile, glazing bite, and installation			requirements.  Manufacturer must provide a window system that can

- incorporate all window configurations used on the project.

- 2.  	Standards:  	Requirements for aluminum windows, terminology and 	standard of performance, and fabrication workmanship are those specified 	and recommended in FGIA/AAMA/WDMA/CSA 101/I.S.2/A440 and The 	Aluminum Association (AA).

- All window units shall be labeled as conforming to FGIA/AAMA/WDMA/CSA 101/I.S.2/A440.  The label shall state the name of the manufacturer, the approved labeling agency and the product designation as specified in FGIA/AAMA/WDMA/CSA 101/I.S.2/A440.

- All testing shall be conducted using FGIA/AAMA/WDMA/CSA 101/I.S.2/A440 Gateway Performance minimum specified test sizes.

1.07	DELIVERY, STORAGE AND HANDLING

A.  Transportation and Handling:  Transport products by methods to avoid product damage, deliver in undamaged condition in manufacturer’s unopened containers or packaging.  Provide equipment and personnel to handle products by method to prevent soiling or damage.  Promptly inspect shipments to assure that products comply with requirements, quantities are correct, and products are undamaged.

B.  Storage and Protection:  Store products in accordance with manufacturer’s instructions, with seals and labels intact and legible.  Store sensitive products in weather-tight enclosures; maintain with temperature and humidity ranges required by manufacturer’s instruction.

1.08	 WARRANTIES

A.  Aluminum Window Warranty

- Products:  Submit a written warranty, executed by the window manufacturer, for the following:

- Framing, sash components, and hardware:  A period of (5) years from the date of manufacture, against defective materials and workmanship, including substantial non-compliance with applicable specification requirements and industry standards, which results in premature failure of the windows or parts, outside of normal wear.

- Insulating glass units:  A period of (10) years from the date of manufacture, against insulating glass seal failure unrelated to glass breakage.

- In the event windows or components are found defective, manufacturer will repair or provide replacements without charge at manufacturer’s option.

- Where applicable, materials which are applied to the face of insulating glass for the purpose of simulating division in glass openings (SDL’s) are warranted against detaching from the glass surface for a period of (5) years.

- Finish:  Refer to Part 2, Section 2.06 “FINISHES” for warranty requirements.

- Warranty for all components must be direct from the manufacturer (non- pass through) and non- prorated for the entire term.  Warranty must be assignable to the non-residential owner, and transferable to subsequent owners through its length.

- Installation:  Submit a written warranty, executed by the window installer, for a period of (1) year from the date of substantial completion, against defective materials or workmanship, including substantial non-compliance with applicable specification requirements, which result in premature failure.

- In the event installation of windows or components is found to be defective, installer will repair or provide replacements without charge at the installer’s option.

PART 2	  PRODUCTS

2.01	MANUFACTURERS

A.  Approved Manufacturers:

- Quaker Window Products Company, Inc. (800) 347-0438

- Single-Hung Window: “T500 SH Series”

- Substitutions: Only pre-approved products specified by the Architect will be acceptable.  Submit the following information with proper documentation as required for pre-bid substitution requests, and at least (10) working days prior to bid date.

- Independent test reports certifying that proposed product is in accordance with, and meets all criteria specified in Section 1.05 “PERFORMANCE REQUIREMENTS”.

- Drawing details of elevations and sections, and samples in accordance with, and as specified in Section 1.03 “SUBMITTALS”.

- Copy of manufacturer’s warranty specified in accordance with, and as specified in Section 1.08 “WARRANTIES”.

- Any additional information requested by the Architect.

2.02 	MATERIALS

A.  Aluminum Members:

- Extruded aluminum prime billet 6063-T6 alloy for primary components, 6063-T6, or 6061-T6 for structural components, all in accordance with (ASTM B221).

B.  Structural Thermal Barrier Construction:

- Frame and sash members shall include a structural thermal barrier, applied in the manufacturer’s facility, using concealed low-conductance poured-in-place polyurethane in a pre-treated cavity.

- After proper curing, the aluminum bridge section must be removed to provide a 1/4” minimum separation between interior and exterior metal surfaces.

- The thermal barrier cavity shall have a manufactured mechanical lock applied consisting of abrading or lancing of the extrusion cavity prior to application of poured-in-place polyurethane.

- Thermal Break Performance Requirements:

- Shear strength:  minimum 2,500 Lbf in accordance with (AAMA TIR-A8).

- Flexural strength:  minimum 19,000 psi in accordance with (AAMA D 790).

- Thermal conductivity of barrier material:  maximum 0.84 BTU-in/(hr-ft2-°F) in accordance with (ASTM C 518).

- Systems employing non-structural thermal barriers, or barrier systems absent of a mechanical lock application are not acceptable.

2.03	MANUFACTURED UNITS

A.  Principal window frame members shall have a minimum 0.062” wall 			      thickness, which includes all hardware mounting webs, and sectional flanges.

B.  Window frame depth shall be 3 1/4” minimum.

C.  Sash Removal: “Side Load” design shall allow for lower sash to be removed 		      from interior by detachment of balances.

D.  Glazing:  Refer to Section 2.05 “GLASS MATERIALS”.

2.04	COMPONENTS

A.  All fasteners, tools, equipment, and other materials necessary for a complete 		     installation shall be furnished by the Contractor.

- Aluminum, nonmagnetic stainless steel, epoxy adhesive, or other materials warranted by the manufacturer to be noncorrosive and compatible with all window members, cladding, trim, hardware, anchors, and other components.

B.  Locking handles, cases, and strikes to be die cast or stainless steel.

C.  Thermoplastic or thermo-set plastic caps, housings, and other components to		      be injection-molded nylon, extruded PVC, or other suitable compound.

D.  Hardware:

- Balances:

Please select one of the following.  A Class 5 balance is recommended for improved manual assist ratio

- Manufacturer’s standard block and tackle balance system which shall be in accordance with (AAMA 101) and (AAMA 902) for Class (1) performance requirements, with a minimum (0.70) Manual Applied Force Ratio (MAF).

- High Performance block and tackle balance system which shall be in accordance with (AAMA 101) and (AAMA 902) for Class (5) performance requirements, with a minimum (0.30) Manual Applied Force Ratio (MAF).

- Sash Lock shall be extruded aluminum, spring loaded gravity latch, located at bottom of lower sash.

- Lift handle shall be extruded, and integral with the bottom sash, and continuous.

E.  Insect Screens:  Provide removable insect screen panel for each movable 		      glazed sash.

Please select one of the following “line 1” items below for screen  type

- Screen Fabric:

- 18 by 18 mesh of 0.012” fabric thickness, .008” yarn diameter.  Comply with FS L-S-125B(4.4.9), Scale 1.

- Screen wire shall contain a durable hydrophobic coating which repels water, and prevents dirt and debris from clinging to the screen

- Screen wire product shall be “Greenguard” certified for low chemical emissions.

- Screen Fabric:  Aluminum:  18 by 16 mesh of 0.013” diameter wire.  Comply with FS-RRW-365, Type VII, except black anodized or “gun metal” coating on wire.

- Screen Frame:  Provide formed or extruded aluminum frames and removable vinyl fabric retainer spline.

- Finish shall match window.

F.  Accessories:

Optional Item

- Safety Device Stops:  For operable windows, provide stops to prevent opening greater than enter opening space measurement.  Stops shall be manufacturer’s standard for intended use, and provided by the manufacturer.   Application shall include tamper-proof screws.

- Sills:  Manufacturer’s standard exterior sills, as shown on Drawings.

Optional Item

- Nailing fin:  Shall be integral to the frame and consist of extruded aluminum as a manufacturer’s standard.

- Sill Flashing:  manufacturer’s standard snap-on type, if required as shown on Drawings.

- Trim:  Manufacturer’s standard interior snap trims, type as shown on Drawings.

- Mullions:  Provide mullions and cover plates as shown, matching window units.  Allow for erection tolerances and provide for movement of window units due to thermal expansion and building deflections, as indicated.  Provide mullions and cover plates capable of withstanding design loads of window units.

Optional Item

- Muntins:

Select one of the Muntin options [ item “a.”] below if required

- Internal Muntins:

- Roll formed aluminum of specified width, located between glass panes within the sealed insulating glass unit.

- Muntins shall have finish to match color of Window Frame.

- Simulated Divided Lite Muntins:

- Composition shall include:

- Exterior applied extruded aluminum Muntin of specified width, continuously adhered to surface of glass with a high performance acrylic adhesive system.

- Roll formed aluminum of specified width, located between glass panes within the sealed insulating glass unit.

- Interior applied extruded aluminum Muntin of specified width, continuously adhered to surface of glass with a high performance acrylic adhesive system.

- Finish of Exterior and Interior applied muntin components shall comply with Section 2.06 “FINISHES”.

- Finish color of Exterior and Interior applied muntins, and internal muntins shall match Window Frame.

Optional Item

- Panning:

- Provide extruded aluminum panning in accordance with (ASTM B221) by Window Manufacturer, type and size as indicated on Drawings.

- Panning joinery shall be cut to fit by Window Manufacturer.

- Finish of Panning components shall comply with Section 2.06 “FINISHES”, and color shall match Aluminum Windows.

Optional Item

- Receptor System / Subframe:

- Provide extruded aluminum, thermally broken Receptor System with Aluminum Windows by Window Manufacturer in accordance with (ASTM B221), as shown on Drawings.

- Finish of Receptor System components shall comply with Section 2.06 “FINISHES”, and color shall match Aluminum Windows.

- Receptor System components shall encompass a complete assembled frame, fastened and sealed independently from window frames prior to window installation according to manufacturer’s instructions.

2.05	GLASS MATERIALS:

Please select one of the [ item “A.”] glass types below which is in accordance with design.  The recommended Low-Emissivity (Low-E) coating options are listed below; however, if the design requires tinted glass, or another specific type of (Low-E), please choose the blank option and add pertinent information

A. Clear Float Glass:  ASTM C1036, Type 1 (transparent glass, flat), 			     Class 1 (clear), Quality q3 (glazing select) as manufactured by one of the 		     following:

- Approved Manufacturers:

- Cardinal Industries

- Approved Substitution by Architect

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),			      Class 1 (clear), Quality q3 (glazing select), with coating type and 		      performance characteristics complying with requirements specified below:

- Low E Coating:  Surface #2 on insulated units.

- Approved Manufacturers:

- “LoE2- 272”; Cardinal Industries

- Approved Substitution by Architect.

- Whole Window U-Value shall be a maximum of 0.42.

- Solar Heat Gain Coefficient shall be a maximum of .33.

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),			     Class 1 (clear), Quality q3 (glazing select), with coating type and performance characteristics complying with requirements specified below:

- Low E Coating:  Surface #2 on insulated units.

- Approved Manufacturers:

- “LoE3- 366”; Cardinal Industries

- Approved Substitution by Architect.

- Whole Window U-Value shall be a maximum of 0.42.

- Solar Heat Gain Coefficient shall be a maximum of .22.

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),			     Class 1 (clear), Quality q3 (glazing select), with coating type and performance characteristics complying with requirements specified below:

- Dual Low E Coating:  Insulated dual pane units.

- Surface #2: “LoE3 -366; Cardinal Industries

- Surface #4: “i89”; Cardinal Industries

- Approved Manufacturers:

- Cardinal Industries

- Approved Substitution by Architect.

- Whole Window U-Value shall be a maximum of 0.38.

- Solar Heat Gain Coefficient shall be a maximum of .21.

A. (Please specify if none of the above items are selected)

B. Tempered Glass:  Condition A (uncoated surfaces), Type 1	 		   	     (transparent glass, flat), Class 1 (clear), Quality q3, clear, fully tempered 		      safety glass (meet requirements of ANSI Z97.1).

- All tempered glass shall conform to ASTM C1048, ANSI Z97.1, and CPSC 16 CFR Part 1201.  Tempered glass shall bear permanent monogram indicating tempered quality.  Fabrication marks on tempered glass shall be located to be concealed in completed installation.

C.  Windows shall be glazed as follows:

- Sound Transmission Class (STC) (ASTM E413):  Provide glazing required for conforming to over all STC ratings as specified for aluminum windows.

- Sound Transmission Class (STC): 28 minimum

For higher sound transmission class requirements, please consult Quaker Window Products (800) 347-0438

D.  Insulating Glass:  Manufacturer’s standard units that comply with specified

quality standards and coatings.

- Provide preassembled units consisting of organically sealed panes of glass enclosing a hermetically sealed dehydrated air space and complying with ASTM E774 for performance classification indicated as well as with other requirements specified for glass characteristics, air space, sealing system, sealant, space material, and desiccants.

For higher sound transmission class requirements, changes may be required to the highlighted values.  Please consult Quaker Window Products (800) 347-0438

- Total Thickness:  1”

- Thickness of Each Pane:  as necessary to meet structural performance criteria.	or specify

- Air Space:  argon gas filled		or specify

- Exterior Pane of Glass:

- Provide tempered glass where shown on Drawings and as required by local codes and ordinances.

- Insulated Unit Sealing System:

- Insulating glass unit spacer system must include a secondary dual seal.  This also applied to solid foam warm edge seal glass spacer systems.

2.06	FINISHES

A. 	Finish of Aluminum Components

1. 	Finish of all exposed areas of aluminum windows and components shall be applied in accordance with the appropriate AAMA Voluntary Guide Specification shown below:

Please select one of the following sets “a., b. & c.” below

- High Performance Organic Powder Coating conforming to (FGIA/AAMA 2604), Voluntary Specification, Performance Requirements and Test Procedures which also meets the following standards:

- Powder Coating resin shall consist of Fluoroethylene Vinyl Ether (FEVE).

- Coatings which require a chrome based liquid primer or pretreatment are not allowed.

- Scratch resistance shall meet or exceed a pencil test of H in accordance with (ASTM D3363-00), “Standard Test Method for Film Hardness by Pencil Test.”

- Abrasion resistance shall meet or exceed a Taber abrasion test of 1000 rotations in accordance with (ASTM D4060-14), “Standard Test Method for Abrasion Resistance of Organic Coatings by the Taber Abraser”.

- Finish Warranty Period: 	10 years from date of manufacture

- Color Selection:  Please select a color

- Superior Performance Organic Powder Coating conforming to (FGIA/AAMA 2605), Voluntary Specification, Performance Requirements and Test Procedures which also meets the following standards:

- Powder Coating resin shall consist of Fluoroethylene Vinyl Ether (FEVE).

- Coatings which require a chrome based liquid primer or pretreatment are not allowed.

- Scratch resistance shall meet or exceed a pencil test of H in accordance with (ASTM D3363-00), “Standard Test Method for Film Hardness by Pencil Test”.

- Abrasion resistance shall meet or exceed a Taber abrasion test of 1000 rotations in accordance with (ASTM D4060-14), “Standard Test Method for Abrasion Resistance of Organic Coatings by the Taber Abraser”.

- Finish Warranty Period:		**15 years from date of manufacture

- Color Selection:  Please select a color

- Electrolytically Deposited Anodic Coating, Class 1, Designation AAM12C21A44 conforming to (FGIA/AAMA 611)

- Finish Warranty Period:		5 years from date of manufacture

- Color Selection: (clear, champagne, light bronze, medium bronze, dark bronze, or black)

- **Optional 20 year warranty is available for Superior Performance Organic Coatings. (AAMA 2605).  Full warranty is for a standard, non-metallic and non-exotic color, and may require a clear topcoat in order to comply with full warranty period.

2.07	FABRICATION

- Fabricate windows allowing for minimum clearances and shim spacing around perimeter of assembly yet enabling installation.

- Rigidly fit joints and corners.  Accurately fit and secure corners tight.  Make corner joints flush, hairline, and weatherproof.  Seal corner joints with sealant.

- Develop drainage holes with moisture pattern to exterior.

- Prepare components to receive anchor devices.  Fabricate anchorage items.

PART 3           EXECUTION

3.01	 EXAMINATION

A.  Site Verification of Conditions:

1. 	Verify that building substrates permit installation of windows according to the manufacturer's instructions, approved shop drawings, calculations and contract documents.

2. 	Do not install windows until unsatisfactory conditions are corrected.

3.02	 INSTALLATION

A. Erection of Aluminum Windows

- Install windows with skilled tradesmen in exact accordance with approved Shop Drawings, Installation Instructions, Specifications, and in accordance with (FGIA/AAMA 101/I.S.2./ A440).

- Windows must be installed plumb, square, and level for proper weathering and operation.  Jambs must not be “sprung”, bowed, or warped during installation.

- Any uncoated aluminum components of Aluminum Window shall be insulated from direct contact with steel, masonry, concrete, or other dissimilar metals by bituminous paint, zinc chromate primer, nonconductive shims, or other suitable insulating materials.

Optional Item

B.   Field Tests

- Field testing procedure of installed windows shall be in accordance with AAMA 502.

- The test pressure used during the field test procedure shall be 2/3 of the rated test pressure of the test specimen in accordance with AAMA 502.  There shall be no optional variances over 2/3 used during testing.

- Initial field testing must be performed prior to no more than 5% of windows have been installed.  or specify

- All field testing expenses shall be at the burden of specify

3.02   ADJUSTING AND CLEANING

A.     After completion of window installation, windows shall be inspected, 	adjusted, put into working order and left clean, free of labels, dirt, or other 	debris.  Protection from this point shall be the responsibility of the General 	Contractor.

END OF SECTION