(QUAKER V250  DOUBLE-HUNG WINDOW)

SECTION 08 53 13 (08560) - VINYL WINDOWS

PART 1	  GENERAL

1.01	SUMMARY

A. 	Section Includes:

1. 	Operating and Fixed Vinyl Window Units

a. Double-Hung Vinyl Windows.

2. 	Glass and Glazing for Vinyl Windows.

3. 	Wood Blocking, Shims, Anchors, Clips, and all accessories necessary for a complete installation furnished and installed.

4. 	All trim and closure pieces

5.  Installation labor, tools, equipment, and services necessary for installation of Vinyl Windows.

B.	Related Sections:

      - Section 07 62 00 (07620) -  Sheet Metal Flashing and Trim

      - Section 07 92 00 (07920) -  Joint Sealants

      - Section 08 32 16 (08320) -  Vinyl Patio Doors

      - Section 08 41 13 (08410) – Aluminum Entrances and Storefront

      - Section 08 80 00 (08800) - Glazing

- 1.02	REFERENCES

A.	Fenestration and Glazing Industry Alliance (FGIA) (a.k.a.) American Architectural Manufacturers Association (AAMA):

      - 101 – “Voluntary Performance Specification for Windows, Skylights and Glass Doors”

      - 502 – “Voluntary Specification for Field Testing of Newly Installed Fenestration Products”

      - 513 – “Voluntary Specification for Standard Laboratory Test Method For Determination Of Forces And Motions Required To Activate Operable Parts Of Operable Windows And Doors In Accessible Spaces”

      - 664 – “Voluntary Specifications for In-Process Quality Control- Requirements for Applicators of Synthetic Interior and Exterior Laminates to Substrates of:  Cellulosic Composite, Cellular Polyvinyl Chloride (PVC), Fiber Reinforced Thermoset, Finished Aluminum, Rigid and Reinforced Thermoplastic, and Wood.

      - 1503 – “Voluntary Test Method for Thermal Transmittance and Condensation Resistance of Windows, Doors and Glazed Wall Sections”

      - 2400 – “Voluntary Specification for Installation of Windows with a Mounting Flange in Stud Frame Construction”

- C.  American National Standards Institute (ANSI) Publications

- Z97.1 – “Performance Specifications and Methods of Test for Safety Glazing Materials Used in Buildings”

- D.  ASTM International (ASTM) Publications:

- C518 – “Standard Test Method for Steady-State Thermal Transmission Properties by Means of the Heat Flow Meter Apparatus”

- C1036 – “Standard Specifications for Flat Glass”

- C1048 – “Standard Specifications for Heat-Treated Flat Glass – Kind HS, Kind FT Coated and Uncoated Glass”

- D3985 – “Standard Test Method for Oxygen Gas Transmission Rate Through Plastic Film and Sheeting Using a Coulometric Sensor”

- E90 – “Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements”

- E283 – “Standard Test Method for Determining the Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen”

- E330 – “Standard Test Method for Structural Performance of Exterior Windows, Curtain Walls, and Doors By Uniform Static Air Pressure Difference”

- E331 – “Standard Test Method for Water Penetration of Exterior Windows, Curtain Walls, and Doors By Uniform Static Air Pressure Difference”

- E413 – “Classification for Rating Sound Insulation”

- E547 – “Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Cyclic Static Air Pressure Differential”

- E774 – “Standard Specification for Sealed Insulating Glass Units”

- F588 - “Standard Test Methods for Measuring the Forced Entry Resistance of Window Assemblies, Excluding Glazing Impact”

- F1249 – “Standard Test Method for Water Vapor Transmission Rate Through Plastic Film and Sheeting Using a Modulated Infrared Sensor”

- E.  National Glass Association (NGA):

- 1.	“GANA Glazing Manual”

- F.  Federal Specifications (FS) Publications:

- 1.	FS-RR-W-365A “Wire Fabric (Insect Screening)”

- G.  Insulating Glass Certification Council (IGCC)

- H.  National Fenestration Ratings Council (NFRC)

- I.  Screen Manufacturers Association (SMA) Publications:

- 1.  	1004 “Specifications for Aluminum Tubular Frame Screens for Windows”

- J.  U.S. Consumer Product Safety Commission (CPSC) Publications:

- 1. 	16 CFR Part 1201 “Safety Standard For Architectural Glazing Materials”

- K.  Window and Door Manufacturers Association (WDMA) Publications:

- 1.	FGIA/AAMA/WDMA  101/I.S.2/NAFS “Voluntary Performance				Specification for Windows, Skylights and Glass Doors”

- 2.  	FGIA/AAMA/WDMA/CSA  101/I.S.2/A440 “Standard/Specification for 			Windows, Doors and Unit Skylights”

1.03	SUBMITTALS

A. 	Submit “Letter of Conformance” in accordance with Section 01 33 00 (01330) – with the following supporting data:

- 1. 	Product data for each type of vinyl window specified, including standard construction details, dimensions of individual components, profiles, finishes, hardware, and accessories.

2. 	Shop drawings for each type of window specified, including ¼-inch scale wall elevations, typical unit elevations at ¾-inch scale details, and full size details of typical composite members.

3.  Samples:  Provide full-size or partial-size sample of window illustrating		glazing system, quality of construction and finish.

4.  Submit certified independent laboratory test reports verifying compliance 	with all test requirements of 1.05 PERFORMANCE				REQUIREMENTS as requested by Architect.

1.04	DEFINITIONS

A. 	Performance grade number, included as part of the FGIA/AAMA/WDMA/CSA 101/I.S.2/A440 product designation code, is actual design pressure in pounds force per square foot used to determine structural test pressure and water test pressure.

1.05    PERFORMANCE REQUIREMENTS

A.   Certify that windows have been tested in accordance with FGIA/American		       Architectural Manufacturers Association (FGIA/AAMA/WDMA) Specification		       for Performance Class specified complying with the following performance 		       standards:

### FGIA/AAMA/WDMA/CSA 101/I.S.2/A440 Performance Requirements:  Provide vinyl windows of the performance class and grade indicated that comply with FGIA/AAMA/WDMA/CSA 101/I.S.2/A440.

- Performance Class:	DH-LC

- Performance Grade:	35

- Structural Test Performance Requirements (ASTM E330):

- Uniform Load Structural Test:  Unit to be tested at 52.50 PSF, both positive and negative, with no glass breakage; damage to make windows inoperable; or permanent deformation of any main frame or ventilating member in excess of 0.2% of its clear span.

- Water Resistance (ASTM E331 and ASTM E547):	No water penetration at test pressure indicated.

- Class DH-LC-35:  5.25 PSF

- Air Leakage (ASTM E283):

- Double-Hung Windows:  Maximum 0.3 CFM per sq./ft. of total exterior surface area, when tested at a static air pressure differential of 1.6 PSF minimum.

- Fixed Windows:  Maximum 0.3 CFM per sq./ft. of total exterior surface area, when tested at a static air pressure difference of 1.6 PSF minimum.

B.  Project Wind Loads:

Please specify design wind pressure, both positive and negative required by the governing building code, as calculated from ANSI A58.1, or as determined by  boundary layer wind tunnel testing.  Minimum for positive and negative pressure = 10.0 p.s.f.

- The system shall be designed to withstand the following loads with respect to the plane of the wall:

- Positive pressure of ___ p.s.f. at non corner zones.

- Negative pressure of ___ p.s.f. at non corner zones.

- Negative pressure of ___ p.s.f. at corner zones.

1.06	QUALITY ASSURANCE

- A.  All window units shall be manufactured by a single source.

- 1.  All windows in any one project must be by the same manufacturer and 	with comparable frame depth, profile, glazing bite, and installation			requirements.  Manufacturer must provide a window system that will

- incorporate all window configurations used on the project.

- 2.  	Standards:  	Requirements for vinyl windows, terminology and 	standard of performance, and fabrication workmanship are those specified 	and recommended in FGIA/AAMA/WDMA/CSA 101/I.S.2/A440.

- All window units shall be labeled as conforming to FGIA/AAMA/WDMA/CSA 101/I.S.2/A440.  The label shall state the name of the manufacturer, the approved labeling agency and the product designation as specified in FGIA/AAMA/WDMA/CSA 101/I.S.2/A440.

- All testing shall be conducted using FGIA/AAMA/WDMA/CSA 101/I.S.2/A440 Gateway Performance minimum specified test sizes.

1.07	DELIVERY, STORAGE AND HANDLING

A.  Transportation and Handling:  Transport products by methods to avoid product damage, deliver in undamaged condition in manufacturer’s unopened containers or packaging.  Provide equipment and personnel to handle products by method to prevent soiling or damage.  Promptly inspect shipments to assure that products comply with requirements, quantities are correct, and products are undamaged.

B.  Storage and Protection:  Store products in accordance with manufacturer’s instructions, with seals and labels intact and legible.  Store sensitive products in weather-tight enclosures; maintain with temperature and humidity ranges required by manufacturer’s instruction.

1.08	 WARRANTIES

A.  Vinyl Window Warranty

- Products:  Submit a written warranty, executed by the window manufacturer, for the following:

- Framing, sash components, and hardware:  A period of (5) years from the date of manufacture, against defective materials, finish, and workmanship, including substantial non-compliance with applicable specification requirements and industry standards, which results in premature failure of the windows or parts, outside of normal wear.

- Insulating glass units:  A period of (10) years from the date of manufacture, against insulating glass seal failure unrelated to glass breakage.

- In the event windows or components are found defective, manufacturer will repair or provide replacements without charge at manufacturer’s option.

- Warranty for all components must be direct from the manufacturer (non- pass through) and non- prorated for the entire term.  Warranty must be assignable to the non-residential owner, and transferable to subsequent owners through its length.

- Installation:  Submit a written warranty, executed by the window installer, for a period of (1) year from the date of substantial completion, against defective materials or workmanship, including substantial non-compliance with applicable specification requirements, which result in premature failure.

- In the event installation of windows or components is found to be defective, installer will repair or provide replacements without charge at the installer’s option.

PART 2	  PRODUCTS

2.01	MANUFACTURERS

A.  Approved Manufacturers:

- Quaker Window Products Company, Inc. (800) 347-0438

- Double-Hung Window: V250 Series

- Substitutions: Only pre-approved products specified by the Architect will be acceptable.  Submit the following information with proper documentation as required for pre-bid substitution requests, and at least (10) working days prior to bid date.

- Independent test reports certifying that proposed product is in accordance with, and meets all criteria specified in Section 1.05 “PERFORMANCE REQUIREMENTS”.

- Drawing details of elevations and sections, and samples in accordance with, and as specified in Section 1.03 “SUBMITTALS”.

- Copy of manufacturer’s warranty specified in accordance with, and as specified in Section 1.08 “WARRANTIES”.

- Any additional information requested by the Architect.

2.02 	MATERIALS

A.  Vinyl Members:

- All extruded vinyl sections shall be of Polyvinyl (vinyl chloride) (uPVC).

2.03	MANUFACTURED UNITS

A.  Principal window frame members and insert members shall be manufactured		      in accordance with FGIA/AAMA/WDMA/CSA 101/I.S.2/A440.

B.  Window frame depth shall be 3 1/4” minimum.

C.  Window frame members shall be designed to include integral J-channel, and		      nail fin as part of the frame extrusion continuous around the perimeter.  		      Corners shall be mitered and welded.

D.  Operation:  Top and bottom sashes shall be equipped with tilting hardware for 	      a “Tilt-in” operation and sash removal ability.

- Top and bottom sash shall include concealed tilt latches which operate freely.

E.  Glazing:  Refer to Section 2.05 “GLASS MATERIALS”.

2.04	COMPONENTS

A.  All fasteners, tools, equipment, and other materials necessary for a complete 		     installation shall be furnished by the Contractor.

B.  Locking handles, cases, and strikes to be stainless steel, or other non-			      corrosive materials compatible with vinyl.

C.  Thermoplastic or thermo-set plastic caps, housings, and other components to		      be injection-molded nylon, extruded PVC, or other suitable compound.

D.  Hardware:

- Balances:

Please select one of the following.  A Class 5 balance is recommended for improved manual assist ratio

- Manufacturer’s standard block and tackle balance system which shall be in accordance with (AAMA 101) and (AAMA 902) for Class (1) performance requirements, with a minimum (0.70) Manual Applied Force Ratio (MAF).

- High Performance block and tackle balance system which shall be in accordance with (AAMA 101) and (AAMA 902) for Class (5) performance requirements, with a minimum (0.30) Manual Applied Force Ratio (MAF).

- Locks:

- Painted zinc die cast sweep locks shall be located at the meeting rail.  (2) locks shall be installed on units wider than 24”.

- Lift handles on bottom sashes shall be integral with the bottom of the bottom sash, and continuous.

E.  Insect Screens:  Provide removable insect screen panel for each movable 		      glazed sash.

Please select one of the following “line 1” items below for screen  type

- Screen Fabric:  Fiberglass:  18 by 18 mesh of 0.012” fabric thickness, .008” yarn diameter.  Comply with FS L-S-125B(4.4.9), Scale 1.

- Screen Fabric:  Aluminum:  18 by 16 mesh of 0.013” diameter wire.  Comply with FS-RRW-365, Type VII, except black anodized or “gun metal” coating on wire.

- Screen Frame:  Provide aluminum frames and removable vinyl fabric retainer spline.

- Finish shall match window.

Please select one of the following “line 3” items below for screen type

- Half Screen:  Screen frame shall sized to fit one sash, and shall have mobility to slide to cover either the bottom or top sash.

- Full Screen:  Screen frame shall cover the full opening over both sashes.  Any cross-bar reinforcements shall be located even with the meeting rail, and shall not be in within the sight of the daylight opening of either sash.

F.  Accessories:

Optional Item

- Safety Device Stops:  For operable windows, provide stops to prevent opening greater than enter opening space measurement.  Stops shall be manufacturer’s standard for intended use, and provided by the manufacturer.

Optional Item

- Muntins:

- Internal Muntins:

- Roll formed aluminum of specified width, located between glass panes within the sealed insulating glass unit.

- Muntins shall have finish to match color of Window Frame..

2.05	GLASS MATERIALS:

Please select one of the [ item “A.”] glass types below which is in accordance with design.  The recommended Low-Emissivity (Low-E) coating options are listed below; however, if the design requires tinted glass, or another specific type of (Low-E), please choose the blank option and add pertinent information

A. Clear Float Glass:  ASTM C1036, Type 1 (transparent glass, flat), 			     Class 1 (clear), Quality q3 (glazing select) as manufactured by one of the 		     following:

- Approved Manufacturers:

- Cardinal Industries

- Approved Substitution by Architect

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),			      Class 1 (clear), Quality q3 (glazing select), with coating type and 		      performance characteristics complying with requirements specified below:

- Low E Coating:  Surface #2 on insulated units.

- Approved Manufacturers:

- “LoE2- 272”; Cardinal Industries

- Approved Substitution by Architect.

- Whole Window U-Value shall be a maximum of 0.29.

- Solar Heat Gain Coefficient shall be a maximum of .33.

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),			     Class 1 (clear), Quality q3 (glazing select), with coating type and performance characteristics complying with requirements specified below:

- Low E Coating:  Surface #2 on insulated units.

- Approved Manufacturers:

- “LoE3- 366”; Cardinal Industries

- Approved Substitution by Architect.

- Whole Window U-Value shall be a maximum of 0.29.

- Solar Heat Gain Coefficient shall be a maximum of .22.

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),			     Class 1 (clear), Quality q3 (glazing select), with coating type and performance characteristics complying with requirements specified below:

- Dual Low E Coating:  Insulated dual pane units.

- Surface #2: “LoE3 -366; Cardinal Industries

- Surface #4: “i89”; Cardinal Industries

- Approved Manufacturers:

- Cardinal Industries

- Approved Substitution by Architect.

- Whole Window U-Value shall be a maximum of 0.25.

- Solar Heat Gain Coefficient shall be a maximum of .21.

A. (Please specify if none of the above items are selected)

B. Tempered Glass:  Condition A (uncoated surfaces), Type 1	 		   	      (transparent glass, flat), Class 1 (clear), Quality q3, clear, fully tempered 		      safety glass (meet requirements of ANSI Z97.1).

- All tempered glass shall conform to ASTM C1048, ANSI Z97.1, and CPSC 16 CFR Part 1201.  Tempered glass shall bear permanent monogram indicating tempered quality.  Fabrication marks on tempered glass shall be located to be concealed in completed installation.

C.  Windows shall be glazed as follows:

- Sound Transmission Class (STC) (ASTM E413):  Provide glazing required for conforming to over all STC ratings as specified for vinyl windows.

For sound transmission class requirements, please consult Quaker Window Products (800) 347-0438

D.  Insulating Glass:  Manufacturer’s standard units that comply with specified

quality standards and coatings.

- Provide preassembled units consisting of organically sealed panes of glass enclosing a hermetically sealed dehydrated air space and complying with ASTM E774 for performance classification indicated as well as with other requirements specified for glass characteristics, air space, sealing system, sealant, space material, and desiccants.

- Total Thickness:  3/4”

- Thickness of Each Pane:  as necessary to meet structural performance criteria.	or specify

- Air Space:  argon gas filled		or specify

For higher sound transmission class requirements, changes may be required to the highlighted values.  Please consult Quaker Window Products (800) 347-0438

- Exterior Pane of Glass:

- Provide tempered glass where shown on Drawings and as required by local codes and ordinances.

- Insulated Unit Sealing System:

- Insulating glass unit spacer system must include a secondary dual seal.  This also applied to solid foam warm edge seal glass spacer systems.

2.06	FINISHES

A. 	Finish of Vinyl Components

1. 	Finish of vinyl windows:  Select one:   White   Beige	Sandstone

Optional exterior coatings:  select one “item no. 2” section with sub-content below

- Special Exterior Paint Finish:

- Reflective Organic Coating conforming to (AAMA 613, AAMA 614, or AAMA 615), Voluntary Specification, Performance Requirements and Test Procedures.

- Paint shall be applied in three coats to all vinyl exterior frame and sash components after frame construction, as to avoid necessary touch-up painting of welded corners, and be baked on at a temperature of 120 to 130 degrees Fahrenheit.

- Welding, and subsequent touching up of pre-painted extrusions in any frame or sash component shall not be allowed.

- Finish Warranty Period:		10 years from date of manufacture

- Color Selection:  Please select a color

- Acrylic Based Exterior Film Application:

- Multi-Layered Laminate Film shall consist of the following heavy-metal -free composition:

- Base film made of composed of colored acrylic (PMMA).

- Two layers of transparent protective surface consisting of Polymethyl Methacrylate (PMMA) middle layer, and a Polyvinylidene Fluoride (PVDF) top layer

- Laminate Film shall conform to (AAMA 664).

- Non acrylic based film products, or any film products containing PVC shall not be allowed.

- Film Warranty Period:	20 years from date of manufacture consisting of a 10 year full replacement warranty with labor and materials included, and an additional 10 year prorated finish material only warranty.

- Color Selection:  Please select a color

- Exterior film shall cover all exposed exterior window frame and sash components.

2.07	FABRICATION

A.  Window frames and sashes shall be assembled in a secure and workmanlike 		      manner to perform as hereinafter specified.  All joints of main frames and 		      sashes shall be mitered and joined neatly and secured by the means of 		      fusion welding.

PART 3           EXECUTION

3.01	 EXAMINATION

A.  Site Verification of Conditions:

1. 	Verify that building substrates permit installation of windows according to the manufacturer's instructions, approved shop drawings, calculations and contract documents.

2. 	Do not install windows until unsatisfactory conditions are corrected.

3.02	 INSTALLATION

A. Erection of Vinyl Windows

- Install windows with skilled tradesmen in exact accordance with approved Shop Drawings, Installation Instructions, Specifications, and in accordance with (FGIA/AAMA 101/I.S.2./ A440).

- Windows must be installed plumb, square, and level for proper weathering and operation.  Jambs must not be “sprung”, bowed, or warped during installation.

Optional Item

B.  Field Tests

- Field testing procedure of installed windows shall be in accordance with AAMA 502.

- The test pressure used during the field test procedure shall be 2/3 of the rated test pressure of the test specimen in accordance with AAMA 502.  There shall be no optional variances over 2/3 used during testing.

- Initial field testing must be performed prior to no more than 5% of windows have been installed.  or specify

- All field testing expenses shall be at the burden of specify

3.02   ADJUSTING AND CLEANING

A.     After completion of window installation, windows shall be inspected, 	adjusted, put into working order and left clean, free of labels, dirt, or other 	debris.  Protection from this point shall be the responsibility of the General 	Contractor.

END OF SECTION