(QUAKER M700 IMPACT RESISTANT ALUMINUM TERRACE DOORS AND FRAMES )

SECTION 08 11 16 – ALUMINUM DOORS AND FRAMES

PART 1	  GENERAL

1.01	SUMMARY

A. 	Section Includes:

1. 	Impact Resistant Aluminum Terrace Door System.

2. 	Perimeter trims.

3. 	Stools and accessories.

4. 	Shims and anchors.

5.  Perimeter seal of Door System.

B.	Related Sections:

      - Section 07 62 00 (07620) -  Sheet Metal Flashing and Trim

      - Section 07 92 00 (07920) -  Joint Sealants

      - Section 08 41 13 (08410) – Aluminum Entrances and Storefront

      - Section 08 51 13 (08520) – Aluminum Windows

      - Section 08 71 00 (08710) – Door Hardware

      - Section 08 80 00 (08800) - Glazing

- 1.02	REFERENCES

A.	 Aluminum Association (AA)

      - DAF-45 – “Designation System for Aluminum Finishes”

- B.	Fenestration and Glazing Industry Alliance (FGIA) (a.k.a.) American Architectural Manufacturers Association (AAMA):

- 1.  1503 – “Voluntary Test Method for Thermal Transmittance and 	Condensation Resistance of Windows, Doors and Glazed Wall Sections”

      - 2604 – “Voluntary Specification, Performance Requirements and Test Procedures for High Performance Organic Coatings on Aluminum Extrusions and Panels”

      - 2605 – “Voluntary Specification, Performance Requirements and Test Procedures for Superior Performance Organic Coatings on Aluminum Extrusions and Panels”

      - 1801 – “Voluntary Specification, Acoustical Rating of Windows, Doors, Skylights and Glazed Wall Sections”

      - 502 – “Voluntary Specification, Newly Installed Fenestration Products”

      - 511 – “Voluntary Specification, Installed Fenestration Products After 6 Months”

      - 513 – “Voluntary Specification for Standard Laboratory Test Method For Determination Of Forces And Motions Required To Activate Operable Parts Of Operable Windows And Doors In Accessible Spaces”

      - CW10 – “Care and Handling of Architectural Aluminum from Shop to Site”

      - 502 – “Voluntary Specification for Field Testing of Newly Installed Fenestration Products”

- C.  ASTM International (ASTM) Publications:

- C518 – “Standard Test Method for Steady-State Thermal Transmission Properties by Means of the Heat Flow Meter Apparatus”

- D3985 – “Standard Test Method for Oxygen Gas Transmission Rate Through Plastic Film and Sheeting Using a Coulometric Sensor”

- E90 – “Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements”

- E283 – “Standard Test Method for Determining the Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen”

- E330 – “Standard Test Method for Structural Performance of Exterior Windows, Curtain Walls, and Doors By Uniform Static Air Pressure Difference”

- E331 – “Standard Test Method for Water Penetration of Exterior Windows, Curtain Walls, and Doors By Uniform Static Air Pressure Difference”

- E547 – “Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Cyclic Static Air Pressure Differential”

- F1249 – “Standard Test Method for Water Vapor Transmission Rate Through Plastic Film and Sheeting Using a Modulated Infrared Sensor”

1.03	SUBMITTALS

A. 	Submit “Letter of Conformance” in accordance with Section 01 33 00 (01330) – with the following supporting data:

- 1. 	Product data.

2. 	Shop drawings.

3.  Samples.

4.	Submit certified independent laboratory test reports verifying compliance 	with all test requirements of 1.04 PERFORMANCE				REQUIREMENTS as requested by Architect.

1.04    PERFORMANCE REQUIREMENTS

A.   Aluminum Door and Frame Performance Requirements:

- Air Infiltration:  The test specimen shall be tested in accordance with ASTM E 283.  Air infiltration rate shall not exceed 0.1 cfm/ft2 of total exterior surface area, when tested at a static air pressure differential of 6.2 PSF minimum.

- Water Resistance (static):  The test specimen shall be tested in accordance with ASTM E 331 for outside glazing.  There shall be no leakage at a minimum static air pressure differential of 12.0 PSF for Out-swing Doors that are closed, and in the locked position, in accordance with ASTM E547.

- Uniform Load:  A static air design load of 135 PSF shall be applied in the positive and negative direction in accordance with ASTM E 330.  There shall be no deflection in excess of L/175 of the span of any framing member at design load.  At structural test load equal to 1.5 times the specified design load, no glass breakage or permanent set in the framing members in excess of 0.2% their clear spans shall occur.

- Thermal Resistance:  When tested in accordance with AAMA 1503, the conductive thermal transmittance (U-factor) shall not be more than .59 BTU/hrs/sf/F for Out-Swing units.

- Condensation Resistance:  When tested in accordance with AAMA 1503, the condensation resistance factor (CRF) shall not be less than 61 for Out-Swing units.

### FGIA/AAMA/WDMA/CSA 101/I.S.2/A440 Performance Requirements:  Provide products of the performance class and grade indicated that comply with FGIA/AAMA/WDMA/CSA 101/I.S.2/A440.

- Performance Class:	AW

- Performance Grade:	70

B.  Windborne Debris Resistance:

- Windows shall be capable of resisting impact from windborne debris in accordance to ASTM E1886 and ASTM E1996.

- Windows shall comply with Florida Building Code 2014, 6.2.2.4 Wind Zone 3, Level D for small and large missile impact.

C.  Project Wind Loads:

Please specify design wind pressure, both positive and negative required by the governing building code, as calculated from ANSI A58.1, or as determined by  boundary layer wind tunnel testing.

- The system shall be designed to withstand the following loads with respect to the plane of the wall:

- Positive pressure of ___ p.s.f. at non corner zones.

- Negative pressure of ___ p.s.f. at non corner zones.

- Negative pressure of ___ p.s.f. at corner zones.

1.05	QUALITY ASSURANCE

- A.  Qualifications:

- Installer Qualifications:  Installer experienced to perform work of this section who has specialized in the installation of work similar to that required for this project, and who is acceptable to product manufacturer.

1.06	DELIVERY, STORAGE AND HANDLING

A.  Transportation and Handling:  Transport products by methods to avoid product damage, deliver in undamaged condition in manufacturer’s unopened containers or packaging.  Provide equipment and personnel to handle products by method to prevent soiling or damage.  Promptly inspect shipments to assure that products comply with requirements, quantities are correct, and products are undamaged.

B.  Storage and Protection:  Store products in accordance with manufacturer’s instructions, with seals and labels intact and legible.  Store sensitive products in weather-tight enclosures; maintain with temperature and humidity ranges required by manufacturer’s instruction.

1.07	 WARRANTIES

A.  Aluminum Doors and Frames Warranty

- Products:  Submit a written warranty, executed by the product manufacturer, for the following:

- Warranty Period:  Framing, door leaf components and hardware:  A period of (5) years from the date of manufacture, against defective materials and workmanship, including substantial non-compliance with applicable specification requirements and industry standards, which results in premature failure of the doors or parts, outside of normal wear.

- Door Corner Construction Warranty:  A lifetime warranty for the life of the door under normal use as determined by Quaker Windows and Door Products.

- Finish:  Refer to Part 2, Section 2.06 “FINISHES” for warranty requirements.

- Insulating glass units:  A period of (10) years from the date of manufacture, against insulating glass seal failure unrelated to glass breakage.

- Where applicable, materials which are applied to the face of insulating glass for the purpose of simulating division in glass openings (SDL’s) are warranted against detaching from the glass surface for a period of (5) years.

- Installation:  Submit a written warranty, executed by the window installer, for a period of (1) year from the date of substantial completion, against defective materials or workmanship, including substantial non-compliance with applicable specification requirements, which result in premature failure.

- In the event installation of Aluminum Doors and Frames or components is found to be defective, installer will repair or provide replacements without charge at the installer’s option.

PART 2	  PRODUCTS

2.01	MANUFACTURERS

A.  Approved Manufacturers:

- Quaker Window Products Company, Inc. (800) 347-0438

- Quaker M700 Series Impact Resistant Aluminum Terrace Doors and Frames.

- Impact Resistance Certification:  Current FPA Number required.

- Substitutions: Only pre-approved products specified by the Architect will be acceptable.  Submit the following information with proper documentation as required for pre-bid substitution requests, and at least (10) working days prior to bid date.

- Independent test reports certifying that proposed product is in accordance with, and meets all criteria specified in Section 1.04 “PERFORMANCE REQUIREMENTS”.

- Drawing details of elevations and sections, and samples in accordance with, and as specified in Section 1.03 “SUBMITTALS”.

- Copy of manufacturer’s warranty specified in accordance with, and as specified in Section 1.07 “WARRANTIES”.

- Any additional information requested by the Architect.

2.02 	MATERIALS

A.  Aluminum Members:

- Extruded aluminum prime billet 6063-T6 alloy for primary components, 6063-T6, or 6061-T6 for structural components, all in accordance with (ASTM B221).

B.  Structural Thermal Barrier Construction:

- Frame and sash members shall include a structural thermal barrier, applied in the manufacturer’s facility, using concealed low-conductance poured-in-place polyurethane in a pre-treated cavity.

- After proper curing, the aluminum bridge section must be removed to provide a 1/4” minimum separation between interior and exterior metal surfaces.

- The thermal barrier cavity shall have a manufactured mechanical lock applied consisting of abrading or lancing of the extrusion cavity prior to application of poured-in-place polyurethane.

- Thermal Break Performance Requirements:

- Shear strength:  minimum 2,500 Lbf in accordance with (AAMA TIR-A8).

- Flexural strength:  minimum 19,000 psi in accordance with (AAMA D 790).

- Thermal conductivity of barrier material:  maximum 0.84 BTU-in/(hr-ft2-°F) in accordance with (ASTM C 518).

- Systems employing non-structural thermal barriers, or barrier systems absent of a mechanical lock application are not acceptable.

2.03	MANUFACTURED UNITS

- Member Wall Thickness:  Outside walls shall have a wall thickness of .125”, and inside wall thickness shall not be less than .070” of framing and door rail members.

- Door frame depth shall be 4 ½” minimum.

- Glazing:  Refer to Section 2.05 “GLASS MATERIALS”.

2.04   FABRICATION

- General:

1   Door corners are to be accurately joined with corner keys and crimped.

- Weather seals shall be open cell, urethane foam core, bonded to a U.V. stabilized polyurethane liner.

B.  Hardware:

1.  Hinging:  Door leaves shall have minimum 1 ½ pairs of mortised 130mm 		long butt hinges painted to match door finish.

- Multi-Point Locking Hardware:

Please select the appropriate style of locking hardware below from one of two manufacturer’s choices, or choose reference to custom hardware

- “Euro Style Lock” gearbox type system, and “800 series” lever handles 	      at exterior and interior by Amesbury Truth or similar.  Locking shall be keyed exterior cylinder and interior thumb turn.

- “Euro Style Lock” gearbox type system, and “800 series” lever handles 	      at exterior and interior by Amesbury Truth or similar.  Locking shall be keyed exterior cylinder and keyed interior cylinder.

- Refer to hardware schedule in Division 7 (Door Hardware) for custom hardware.

Optional Item

C.  Muntins:

- Simulated Divided Lite Muntins:

- Composition shall include:

- Exterior applied extruded aluminum Muntin of specified width, continuously adhered to surface of glass with a high performance acrylic adhesive system.

- Interior applied extruded aluminum Muntin of specified width, continuously adhered to surface of glass with a high performance acrylic adhesive system.

- Finish of Exterior and Interior applied muntin components shall comply with Section 2.06 “FINISHES”.

- Finish color of Exterior and Interior applied muntins shall match Door Frame.

2.05	GLASS MATERIALS:

Please select one of the [ item “A.”] glass types below which is in accordance with design.  The recommended Low-Emissivity (Low-E) coating options are listed below; however, if the design requires tinted glass, or another specific type of (Low-E), please choose the blank option and add pertinent information

A. Clear Float Glass:  ASTM C1036, Type 1 (transparent glass, flat), 			     Class 1 (clear), Quality q3 (glazing select) as manufactured by one of the 		     following:

- Approved Manufacturers:

- Cardinal Industries

- Approved Substitution by Architect

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),			      Class 1 (clear), Quality q3 (glazing select), with coating type and 		      performance characteristics complying with requirements specified below:

- Low E Coating:  Surface #2 on insulated units.

- Approved Manufacturers:

- “LoE2- 272”; Cardinal Industries

- Approved Substitution by Architect.

- Whole Window U-Value shall be a maximum of 0.40.

- Solar Heat Gain Coefficient shall be a maximum of .28.

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),			     Class 1 (clear), Quality q3 (glazing select), with coating type and performance characteristics complying with requirements specified below:

- Low E Coating:  Surface #2 on insulated units.

- Approved Manufacturers:

- “LoE3- 366”; Cardinal Industries

- Approved Substitution by Architect.

- Whole Window U-Value shall be a maximum of 0.39.

- Solar Heat Gain Coefficient shall be a maximum of .19.

- Coated Low Emissivity Glass:  Type 1 (transparent glass, flat),			     Class 1 (clear), Quality q3 (glazing select), with coating type and performance characteristics complying with requirements specified below:

- Dual Low E Coating:  Insulated dual pane units.

- Surface #2: “LoE3 -366; Cardinal Industries

- Surface #4: “i89”; Cardinal Industries

- Approved Manufacturers:

- Cardinal Industries

- Approved Substitution by Architect.

- Whole Window U-Value shall be a maximum of 0.35.

- Solar Heat Gain Coefficient shall be a maximum of .19.

A. (Please specify if none of the above items are selected)

B. Tempered Glass:  Condition A (uncoated surfaces), Type 1	 		   	      (transparent glass, flat), Class 1 (clear), Quality q3, clear, fully tempered 		      safety glass (meet requirements of ANSI Z97.1).

- All tempered glass shall conform to ASTM C1048, ANSI Z97.1, and CPSC 16 CFR Part 1201.  Tempered glass shall bear permanent monogram indicating tempered quality.  Fabrication marks on tempered glass shall be located to be concealed in completed installation.

C.  Doors shall be glazed as follows:

- Sound Transmission Class (STC) (ASTM E413):  Provide glazing required for conforming to overall STC ratings as specified for Aluminum Terrace Doors.

For sound transmission class requirements, please consult Quaker Window Products (800) 347-0438

D.  Insulating Glass:  Manufacturer’s standard units that comply with specified

quality standards and coatings.

- Provide preassembled units consisting of organically sealed panes of glass enclosing a hermetically sealed dehydrated air space and complying with ASTM E774 for performance classification indicated as well as with other requirements specified for glass characteristics, air space, sealing system, sealant, space material, and desiccants.

- Total Thickness:  1”

- Unit Composition:

  - Inboard lite:  As required to meet ASTM E1886 and AST E1996 Windborne Debris Resistance.

  - Air Space:  Argon gas filled

  - Inboard lite:  Laminated as required to meet ASTM E1886 and ASTM E1996 Windborne Debris Resistance.

- Insulated Unit Sealing System:

- Insulating glass unit spacer system must include a secondary dual seal.  This also applied to solid foam warm edge seal glass spacer systems.

2.06	FINISHES

A.  Finish of Aluminum Components

1. 	Finish of all exposed areas of aluminum windows and components shall be applied in accordance with the appropriate AAMA Voluntary Guide Specification shown below:

Please select one of the following sets “a., b. & c.” below

- High Performance Organic Powder Coating conforming to (FGIA/AAMA 2604), Voluntary Specification, Performance Requirements and Test Procedures which also meets the following standards:

- Powder Coating resin shall consist of Fluoroethylene Vinyl Ether (FEVE).

- Coatings which require a chrome based liquid primer or pretreatment are not allowed.

- Scratch resistance shall meet or exceed a pencil test of H in accordance with (ASTM D3363-00), “Standard Test Method for Film Hardness by Pencil Test.”

- Abrasion resistance shall meet or exceed a Taber abrasion test of 1000 rotations in accordance with (ASTM D4060-14), “Standard Test Method for Abrasion Resistance of Organic Coatings by the Taber Abraser”.

- Finish Warranty Period: 	10 years from date of manufacture

- Color Selection:  Please select a color

- Superior Performance Organic Powder Coating conforming to (FGIA/AAMA 2605), Voluntary Specification, Performance Requirements and Test Procedures which also meets the following standards:

- Powder Coating resin shall consist of Fluoroethylene Vinyl Ether (FEVE).

- Coatings which require a chrome based liquid primer or pretreatment are not allowed.

- Scratch resistance shall meet or exceed a pencil test of H in accordance with (ASTM D3363-00), “Standard Test Method for Film Hardness by Pencil Test”.

- Abrasion resistance shall meet or exceed a Taber abrasion test of 1000 rotations in accordance with (ASTM D4060-14), “Standard Test Method for Abrasion Resistance of Organic Coatings by the Taber Abraser”.

- Finish Warranty Period:		**15 years from date of manufacture

- Color Selection:  Please select a color

- Electrolytically Deposited Anodic Coating, Class 1, Designation AAM12C21A44 conforming to (FGIA/AAMA 611)

- Finish Warranty Period:		5 years from date of manufacture

- Color Selection: (clear, champagne, light bronze, medium bronze, dark bronze, or black)

- **Optional 20 year warranty is available for Superior Performance Organic Coatings. (AAMA 2605).  Full warranty is for a standard, non-metallic and non-exotic color, and may require a clear topcoat in order to comply with full warranty period.

PART 3           EXECUTION

3.01	 EXAMINATION

A.  Site Verification of Conditions:

1. 	Verify that building substrates permit installation of Aluminum Doors and Frames according to the manufacturer's instructions, approved shop drawings, calculations and contract documents.

2. 	Do not install Aluminum Doors and Frames until unsatisfactory conditions are corrected.

3.02	 INSTALLATION

A. Erection of Aluminum Doors and Frames:

- Install Aluminum Doors and Frames with skilled tradesmen in exact accordance with approved Shop Drawings, Installation Instructions, Specifications, and in accordance with requirements.

- Fabricated units must be installed plumb, square, and level for proper weathering and operation without warp, or rack of frame.

- Any uncoated aluminum components of Aluminum Doors and Frames shall be insulated from direct contact with steel, masonry, concrete, or other dissimilar metals by bituminous paint, zinc chromate primer, nonconductive shims, or other suitable insulating materials.

- Related Products Installation Requirements:

- Perimeter Sealants: Refer to Division 7 Sealant Section.

Optional Item

C.   Field Tests

- Field testing procedure of installed windows shall be in accordance with AAMA 502.

- The test pressure used during the field test procedure shall be 2/3 of the rated test pressure of the test specimen in accordance with AAMA 502.  There shall be no optional variances over 2/3 used during testing.

- Initial field testing must be performed prior to no more than 5% of windows have been installed.  or specify

- All field testing expenses shall be at the burden of specify

3.02   PROTECTION AND CLEANING

- After completion of Aluminum Doors and Frame installation, installed		units shall be inspected, adjusted, put into working order and left clean, free		of labels, dirt, or other debris.  Protection from this point shall be the			responsibility of the General Contractor.

- Protection:  General Contractor shall protect product’s finish surfaces from damage during construction.  Protect all components from damage from grinding and polishing compounds, plaster, lime, acid, cement, or other harmful contaminants.

END OF SECTION